South Dakota School of Mines & Technology

Department of Materials and Metallurgical Engineering

MET 321	Homework #11	Due March 23, 2001

# Alternative Standard States Homework Exercise

Determine G° at 1600°C for the

Si + O2 = SiO2

For the following combinations of standard states:

Combination	Si		 O2		SiO2

1	pure l	1 atm	pure l

2	    "	 "	pure  cristobalite, high

3	    "	1 torr	pure l

4	    "	   "	pure  cristobalite, high

5	1wt %	1 atm	pure l

6	   "	 "	pure  cristobalite, high

7	   "	1 torr	pure l

8	   "	"	pure  cristobalite, high

Data:

Si(l) + 02(g) = Si02 (-cristobalite ,high)

G\s\up6(o)= - 218,600 + 47.40 T cal/gmole

Si(l) in iron;	\s\up2(°)1873°K = 0.00132*

log fSi = 0.07 [% Si]*

Atomic Wt Si = 28.09

H°fusion, ( -cristobalite ,high)= 2,290 cal/gmole

Tfusion,( -cristobalite ,high) = 1,996 K

* Data obtained from Hultgren et al. and JANAF